# Brown Glacier Data Report

## GPS measurements

All the surveying in the 2000 field season was done with GPS methods. A temporary base station was set up on an obvious isolated rock just east of the location of the temporary hut. The base station was marked with a chiseled and painted cross and a rock cairn. Leica Geosystems ® receivers were used in the field and Ski Pro 1.1 ® software to reduce the data. The base station’s location was calculated by solving the baseline to the summer station at Atlas Cove, which AUSLIG calculated to be at 53 1’ 7.0222” S, 73 23’ 36.2753” E, 44.2050 m. Heights are with reference to the WGS84 ellipsoid. Sea level is about 40 m above ellipsoid height at Heard Island. The position of the Brown local base station was at 53 4’ 50.6748” S, 73 40’ 28.3220” E, 68.1100 m.

We ran half hourly fast static sessions at a total of 19 points along a longitudinal and two transverse profiles to determine glacier velocities over two epochs. In addition a GPS station was left on the glacier at marker BG35 and collected data four times a day. We also collected several kinematic profiles, along the lagoon shoreline, the moraines, the glacier terminus and on the glacier itself. All those data were reduced to determine the position in the WGS84 system. They were also transformed into UTM Zone 43 (spheroid GRS80) using the coordinate transformation utility in Ski Pro ®, so they could be compared to an existing DEM of Heard Island. The elevation above sea level was obtained by subtracting 40 m off the ellipsoid height; the same procedure was used in the creation of the DEM.

## Glacier velocities

Figure 1 shows the velocity vectors obtained at all index sites over two epochs (27/28 October to 7/9 November and 7/9 November to 21/22 November). The index sites were named BG05 at the head of the glacier to BG50 at the terminus and are separated by approximately 500 m. The circles attached to the velocity vectors indicate estimated errors. With the exception of BG20 and BG25, all velocities are equal over the two epochs within the estimated error. At BG20 and BG25 flow directions differ in a way that is not within the expected GPS error, but seems unrealistic anyway. The BG05 velocity could be in error by as much as 0.03 m/d, because the pole tilted substantially.

Figure 2 shows the velocity magnitude as a function of distance along an approximate flowline. The measured speeds are the same within the margin of error over the two epochs. There are no indications of a speed up as spring approaches. On the other hand, the first signs of widespread melting did not occur during these observation periods.

Two transverse profiles were measured, one at BG35 and one at BG40. Their purpose was to get a better estimate of the total ice flux near the equilibrium line, which we judged to be near BG40. The BG35 profile was chosen because it was less crevassed and easier to work at. The transverse profiles show that the fastest moving ice is somewhat to the East of the chosen index sites. The radar surveys discussed below also show that the deepest part of the channel is situated to the East of the BG35 index site.

Figure 1: Velocity vectors from two epochs for all index sites. The circle on the right indicates the position of the GPS base station. Circles at the end of the velocity vectors indicate estimated errors.

One GPS station was left near the BG35 index site to record data four times daily. There are substantial data gaps due to power supply problems. The calculated velocity is shown in Figure 4. It has a relatively large error because of the short times between most observations. The velocity is constant within the error margins and does not show any diurnal variations, or a consistent increase with time. The average horizontal velocity was 0.117 m/d, in good agreement with the velocity shown in Figure 2.

## Vertical velocity

The vertical velocity  can be divided into several components. It is useful to remove the component due to the downslope movement. This leads to the emergence velocity (Paterson, 1994):

where  is the local surface slope, and the horizontal velocity. If it is assumed that

The average vertical velocity  was –0.049 m/d. This can be corrected by taking into account the surface parallel downslope movement to obtain the emergence velocity

where  is the local surface slope, which was measured to be about 11. Allowing for an error in surface slope of 2, an emergence velocity of –0.026  0.004 m/d or 9.5  1.5 m/a results. The emergence velocity is the combined effect of mass balance  (positive mass balance leads to negative emergence velocity) and surface changes :

A snow pit dug on 31 October revealed 0.7 m ice equivalent winter accumulation. There was additional snow fall after that date, but not substantial amounts. This indicates that the surface at BG35 is drawn down by at least 7 m/a, possibly more!

Figure 2: Longitudinal velocity profile. Distance is along an approximate flowline, starting at the highest index sites, about 500 m from the glacier head. The blue line shows velocities over the first epoch, and the green one over the second one.

Figure 3: Transverse velocity profiles at BG35 (one epoch) and BG40 (two epochs). Distance is in m from the BG35 and BG40 respectively.

Figure 4: Short term velocities of BG35.

Figure 5: Vertical position of BG35 marker.

Figure 6:  Emergence velocity for markers at the index sites.