# BROWN GLACIER MORPHOLOGY

The best existing map of Heard Island is a digital elevation model (DEM) created from a pair of RADARSAT images. Elevations were evaluated on a 50 m by 50 m grid. The accuracy of the map is not sufficient to estimate volume changes, however. We therefore collected several kinematic GPS profiles on and around Brown Glacier (Fig. Brown DEM/map). The map in Figure (Brown DEM/map) was created by adjusting the existing DEM (accuracy 20 m) to the GPS profiles (accuracy 0.5 m).

Brown Glacier is a steep (average slope 12) and thin glacier. We measured ice thickness in several places using a radio echo sounding (RES) system (Narod and Clarke, 1994). A full transverse profile was obtained at BG35 and spot depths at BG30, BG45, and BG50 (Fig. RES). We also recorded a maximum depth of the proglacial lagoon of just over 50 m.

The exact extent of the glacier is difficult to define, as the ice divides with the Compton Glacier to the west, and the mostly inactive remnants of a glacier to the east of Brown are not very obvious. We used our local DEM (Fig. DEM/map) and field observations to define the glacier outline. From that and an interpolation of the few depth measurements (Fig. RES) we obtain a rough estimate of 9108 m3 for the current (year 2000) volume of the glacier, and 3.7106 m2 for its area.

On 1947 photographs the glacier is seen to be an active tidewater glacier (Fig. 1947photo). We estimated the volume change of the glacier in the last 53 years by interpolating a surface between the surveyed old lateral moraines, the sea shore and a profile at BG35, and calculating the difference to the present day surface. This assumes that the volume change is negligible in the upper part of the glacier. We obtain a volume change of 1.6108 m3, or about 15%, and an area change of 3.7106 m2, or about 33%. The large area change is partially due to the loss of ice on the orographic left of the glacier (Fig. DEM/map).